120/01		N13/PANEL CUBICLES – UNFRAMED

- Drawing reference(s):

- Manufacturer and reference: Rearo Laminates, 29 Loanbank Quadrant, Glasgow, G51 3HZ.

- Tel 0141 440 0800. E-mail commercial@rearo.co.uk. URL www.rearocommercial.co.uk.

- Product: ‘Aspire MR MFC’

- Board/panel type: 600kg/m³ grade MR MFC (moisture resistant Melamine Faced Chipboard).

- Panel thickness: 19mm nominal.

- Edge treatment: 2mm matching ABS.

- Range height: 1950mm FFL.

- Panel height: 1800mm.

- Depth: 1525mm (1500mm deep partition panels).

- Floor clearance: 150mm.

- Pedestal adjustment: ± 20mm.

- Pedestal: Aluminium, painted Nickel finish.

- Headrail: Aluminium slimline ‘U’ channel with no visible fixings. Painted Nickel finish.

- Hinges: Aluminium lift off, gravity hinges with front caps. Painted Nickel finish.

- Indicator bolt and keeper: Aluminium slide action lock with no visible fixings. Circular flush fitting faceplate and emergency release facility. Painted Nickel finish.

- Wall fixings: Aluminium ‘U’ brackets painted Nickel finish with bolt through fixings.

- Colour choice: 9 colours from the Aspire colour range.

- Notes

- All cubicle panels are manufactured bespoke but some panels may require scribing on site to suit.